SCHOOL OF ENVIRONMENTAL SCIENCES

VERC PROJECT

MAHATMA GANDHI UNIVERSITY

SES/MGU/VERC/F-79/E /2018						Date:02.02.2018

QUOTATION NOTICE

Sealed quotations in original are invited for the supply of following items. The cover containing the quotations should be superscribed “Quotation for the supply of Minor Equipment for VERC project”and the same should be addressed to TheDirector, VERC project, School of Environmental Sciences, M.G. University, P.D Hills, Kottayam – 686 560.Last date for receipt of sealed quotations: 3 pm on 15/02/2018. The quotation will be opened on the same day at 3.30pm.

| S. No | Instruments/other items | Specification | Size/capacity | Numbers |
| --- | --- | --- | --- | --- |
|  | Ultrasonic bath | Based on the principle of High frequency electrical energy converted into ultrasound waves by means of ultrasonic transducers. Complete with heater and digital temperature controller. Material : Stainless Steel Temperature Range : Ambient + 5 to 70˚C Operating frequency 33±3 KHz, for all the general purpose. Input voltage range 170V AC – 270V AC, 50 Hz. Micro controller based timer range 0 -15 minutes up to 3.5 hrs. Thermostatic heating and Digital temperature controller. Tank size: 300Lx150Bx100H (mm) | 3.0 to 3.5 L | 1 |
|  | 4” diameter sieve set (stainless steel) without shaker | Stainless steel sieve of different sizes (2mm, 1mm, 200µm, 50 µm, and 20 µm)- height 5.5 cm and diameter 10 cm and lid and pan | 2mm,1mm 200µm,50 µm, and 20 µm | 2 of each size |
|  | Sieve set with suitable Sieve Shakers | A set of Stainless steel ASTM sieves with a normal diameter (8 inch) and varying pore size (2000, 1410,1000, 710, 500,350, 250, 210,177, 125, 88,63 microns) – with lid and pan Sieve Shakers – suitable for the sieve set mentioned above (Preferably same make) . Electrical requirements:100-240 V, 50/60 Hz, Accommodate up to 6-10 sieves. With time setter in the range of 10 min to 2 hours |  | 1 set |
|  | - Hot air oven | Three Perforated removable stainless steel trays Power supply: 230VAC 50Hz , Power: 1500W Temperature: 5°C to 200°C, Temperature resolution: 0.1°C, Temperature accuracy : ±1°C Temperature control: Digital PID temperature control | 18”x18”x18” | 1 |
|  | - Digital magnetic stirrer with Hot plate | Digital Magnetic Stirrer with Hot Plate, Weight Capacity : 11 Kg/5 Ltr, Temperature : 50°C to 500°C | 18 x 18 cm | 1 |
|  | - Digital Hotplate | Hot plate with controlled temperature Weight Capacity: 16 Kg, Temperature: 50°C to 370°C | 26 x 26 cm | 1 |
|  | Digital pH and EC meter | Digital pH and EC meter (with two input probs) | Two inputs for the simultaneous measurement of pH/mV/ISE and conductivity | 2 |
|  | Rotary mixer | Speed range : 5-50RPM , LED speed display Timer set: 0-99 hrs/1-59 min, Rotating bars for tube dia: 10-30mm with two additional rotating bar of different size (20x15ml, and 12x50ml) | - 20x15ml 12x50ml | 1 1 1 |
|  | Magnetic stirrer bar/ pellet | Round magnetic stirrer bead with pivot ring (Magnet completely enclosed in Teflon) | 8 x 22 mm 8 x 40 mm | 2 2 |
|  | # Digital Vernier Caliper | # 150 mm, 6 inch LCD Digital Vernier Caliper | 6 inches | 6 |
|  | - Digital Stopwatch | 1 lines display, 1/100 seconds, plastic case: 64x53x19mm , 1 memories | 18x37mm | 6 |
|  | Mortar and Pestle | # Mortar and Pestle made of solid marble | 10- 15 cm dia | 2 |
|  | GPS | # Base map loading : Worldwide base map # Display : Minimum 1.5 x 1.7 and 2.2" diag inch monochrome display, easy to read in any light, transflective # Satellite reception capability : GPS and GLONASS satellites # Battery : Minimum 24 hour life with rechargeable batteries # Accuracy : Minimum 3 m. |  | 1 |

Terms and Conditions

1.	The cost of items should include all taxes.

2.   The University will not be responsible for postal delay or loss in transit.

3.	The undersigned reserves every right to reject or accept any or all quotations without assigning any reason, thereof.

4.	The quotation should reach the office of School of Environmental Sciences with all details before 3 pm on 15/02/2018 and will be opened at 3.30 p.m. on 15/02/2018.

5.	The quotation may be sent to the Director, VERC Project, School of Environmental Sciences, Mahatma Gandhi University, Priyadarshini Hills P.O.,  Kottayam – 686 560

6.  The quotations without observing the above specifications, terms and conditions will be rejected.

Sd/-

Director